Operation deployment 1: ______________________________________

Obtain ISO installation medium of Windows Server 2016 from the evaluation download site of Microsoft and disc installation;

https://www.microsoft.com/en-us/evalcenter/evaluate-windows-server-2016

Disc ISO file to DVD optical disc or USB for installation

If you don’t use new computer equipment for installation, please ensure the important data installed on computer disc has been backed up, because the data on the computer hard disk will be erased in the installation process.

Select standard version or data center version for installation according to the information authorized formally.

Notes: Must Select Desktop Experience installation mode.

After installation for the first time, set password of Administrator, and the set password must meet a certain complexity.

Attentions:

1:	The minimum hardware configuration of system installation must meet the performance of dual-core 2G memory.

2:	The host hardware CPU should support DEP property (Data Execution Protection), otherwise, Windows Server 2016 cannot run normally after installation.

Most of DEP properties are defaulted to be opened by BIOS, if your host CPU supports DEP, but please contact the equipment manufacturer for consultation if the problems are appeared in the actual installation.

3:	If you hope to install Windows Server 2016 in the virtualization environment of server, besides hardware CPU and BIOS support DPE and virtualization instruction, and the virtualization software is used in the software version published after January 2016 or the corresponding function upgrade package is provided to formally support Windows Server 2016.

Therefore, it is not feasible to install and run Windows Server 2016 on Hyper-V of Windows Server 2008 R2.

4:	If your environment is AMD CPU structure server + VMware ESX5i; Because VMWare ESxi supports the official version of ESXi6 update 2 of Windows Server 2016, VMWareESxi 6 has not supported all AMD CPUs, if VMware ESX6i is upgraded or installed compulsorily on AMD server, the unknown phenomena of purple screen and black screen of start and running failure will be appeared;

Therefore, in this case, you should select the installation environment in addition.

5:	There are two desktop modes for Windows Server 2016 MultiPoint application; i.e., session-based desktop mode and VDI desktop mode; if you hope to use VDI application mode of Windows Server 2016 MultiPoint, you must install Windows Server 2016 on the physical server.

6:	Then hardware (* including server virtualization) display card of host must provide the drives of display card and network card compatible with WIN10 X64.

7:	Press PS/2 button mouse or USB button mouse, but not use button mouse combinations of PS/2 and USB according to the requirement on the host.

In addition, the monitor resolution on the host is >=1024*768 resolution mode

8:	After installation, when zero terminals is applied, the host should be connected with display or KVM display connector, otherwise after the host is restarted, the system cannot normally enter the zero terminal work mode.

9:	The different Windows Server 2016 installation versions are used within 30 or 180 days period, and all use risk information will be reminded in the lower right corner of system in case of exceeding the probation period, you can run the command: slmgr.vbs –rearm extends the probation period.

Operation steps 2:___________________________________________________________________

- After the system is installed, all drives of chipset, display card and network card of host are installed correctly.

- Click Windows Server 2016 start menu – server administrator for MultiPoint role installation

- Click instrument board – administer – add role function

- Appear role addition and guide interface, select Next to continue installation

- Select installation type – remote desktop service installation

- Select installation multipoint service

- Select the installed server, click Next to install

- Select Restart the destination server automatically if required for deployment.

- Wait for deployment process, and restart the host automatically after deployment.

- After installation, appear the following login interface, log in for the first time, long press “B” button for 5-20s on keyboard.

*	It is suggested to use PS/2 button mouse in deployment, if there is no response after using USB button mouse mode to press “B” button for a long time (more than 30s), please try to pull and insert USB button mouse, replace the insert port of USB button mouse or replace the operating method of button mouse model.

*	Prohibit using combined application mode of PS/2+USB button mouse under MultiPoint Services

- After entering system, remind completing deployment.

- Two key programs are installed by Windows Server 2016 of Centerm C75 network zero terminal;

*	GenericWMSZeroClientService_Setup_14_0904_S0153_G1051_Cert.exe

*	SetupSgfxWmsFbr.exe

Firstly install GenericWMSZeroClientService Setup_14_0904_S0153_G1051_Cert.exe

Then install SetupSgfxWmsFbr.exe; Restart after installation

- Restart host after two key programs are installed.

Operation steps 3:___________________________________________________________________

Configure network IP address of host.

The administration function program of Windows Multipoint Server 2016 includes two main program modules:

*	MultiPoint Manager

*	MultiPoint Dashboard

The specific relevant program file path is located in C:\Program Files\Windows MultiPoint Server\, where MultiPoint Manager is the management software of zero terminal application environment, and MultiPoint Dashboard is the behavior management software of specific zero terminal desktop.

- Click Start - MultiPoint Manager.

- After clicking MultiPoint Manager, appear administration interface, equip with free desktop authorization for 120 days after being defaulted to install.

*	If it is deployed formally, please deploy according to the reminder after clicking and adding client-side access license.

- Click users in MultiPoint Manager to create the user account number.

Create different account number types according to different user roles.

For example, create Standard user account number for students.

For example, create MultiPoint Dashboard user account number for teacher.

For example, create Administrative user account number for host administrator or IT administrator.

- Click network station in MultiPoint Manager, deploy Centerm C75 zero terminal and Windows Server 2016 host in one physical subnetwork environment, then start Centerm C75 zero terminal, and wait for discovery equipment of MultiPoint Manager.

If DHCP is set in the network, after IP address distributed by DHCP obtained from Centerm C75 zero terminals, Centerm C75 zero terminal equipment can be discovered rapidly by the host.

If DHCP is not set in network, after electrically starting Centerm C75 zero terminals, wait for 30-60s, zero terminals can be distributed to one temporary IP address, and it will be discovered by the host.

The temporary IP is used in zero terminals, and the zero terminals can be found in the host; But the zero terminal cannot be connected to host.

- After Centerm C75 zero terminals is found in the host, if IP on zero terminal is temporary IP address, right click the list, click change network setting, cancel DHCP address mode, and set correct fixed IP address for zero terminal.

- Confirm whether IP address of Centerm C75 zero terminal is set normally.

- After correct IP address of Centerm C75 zero terminal is set, select zero terminal list, and the option assigned to the server is appeared.

- Click the option assigned to the server, Centerm C75 zero terminal will establish communication connection with host; after establishing connection, remind “connected” state in the status bar.

- The traditional Windows login interface is appeared in Centerm C75 zero terminals; enter the corresponding Windows desktop experience environment after inputting correct user name and password.

Under defaulted configuration, the use reminder is appeared when the common user enters the system for the first time; click Continue using this computer to compute confirmation of desktop use.

Operation steps 4:  _________________________________________________________________

- Use Windows administrator account number or MultiPoint Dashboard permission account number to log in Windows Server 2016 desktop on consol or Centerm C75 desktop

The specific relevant program file path is located in C:\Program Files\Windows MultiPoint Server\, where MultiPoint Manager is the management software of zero terminal application environment, and MultiPoint Dashboard is the behavior management software of specific zero terminal desktop.

- Directly run Dashboard automatically after MultiPoint Dashboard permission account number logs in

- In MultiPoint Dashboard administration interface, MultiPoint Dashboard user (such as teacher) can apply all behavior controls or distance teachings for the user (such as student) desktop connected to host.

Appendix:______________________________________________________________________________

The user password system strategy of Windows Server 2016 defines the complexity, and it may be inconvenient to the account number creation and user use under the application of simple scenes of education training, and it can be modified through configuring the group strategy of system.

- Click Start -Windows PowerShell

- Callout Local Group Policy Editors

- Configure Password must meet complexity requirements to Disable

- Input application of gpupdate.exe/force execution strategy

- Create user confirmation in MultiPoint Manager