README.DOC - \Not_supp\Driver\LPT1

==================================

There are six files and one sub-directory (lpsource) in this directory.

The files are:

1.readme.doc

2.doc.ps

3.dumb_int.sh

4.install.sh

5.instmsgs

6.mycc

--readme.doc

This file.

--doc.ps

This is a PostScript data file containing 04 ASCII codes at its beginning and

the end.  It is used for testing the PostScript printer connected with

PrintServer.

--dumb_int.sh

dumb_int.sh is a Bourne Shell script.  It is an interface program of a dumb

printer for System V Unix.  It pipes the banner page and data files to the

PSfilter program.

--install.sh

install.sh is provided to help user attach a printer on the PrintServer to the

UNIX system so that the printer acts as a local printer of the UNIX host.

install.sh attaches one printer at a time.  If you have more than one printer

connected to the PrintServer, you must run install.sh for each printer.

If no paramaters are passed with install.sh, the install.sh program will go

through the interactive procedure, which will ask the questions about the

configuration of the printer one by one.

You can install PrintServer in a quick way with the default configuration

by passing the necessary arguments to install.sh.

The following arguments and options can be used:

(Options are shown in [ ], but the [ ] themselves are not typed in.

All arguments and options be on one line.)

install.sh printer_name PS_HostName

[pn=port_number]

[im=interface_model]

[d=delay_minutes]

[text]

Where:

*printer_name is the name of the printer to be added.

*PS_HostName is the host name of the print server device.

*interface_model is used only on system V, it is a model of the interface

program of the printer.

*port_number can be one of the following, depending on your PrintServer model:

1               parallel port 1

2               parallel port 2

3               parallel port 3

*delay_minutes is the number of minutes allowed for printer faults before

the print job being aborted.  If delay_minutes is 0, the print job

will be aborted immediately after printer faults.  If it is -1, the

PSfilter program will keep waiting for the recovery of the printer

faults.

*text is a parameter to instruct PSfilter to convert LineFeed to

CarriageReturn + LineFeed.

port_number, interface_model, delay minutes and text mode are optional.

If they are not specified, the parallel port 1,  dumb_int.sh, -1(infinite

delay minutes) and non_text mode will be used.

--instmsgs

Contains all output messages of install.sh

--mycc

The mycc program is used to compile the psfilter.c source code.  Its output

execution file is PSfilter in the subdirectory ./lpsource.

Normally, you don't have to run mycc; it will be invoked by install.sh.

To run mycc, type:

./mycc

It will be invoked by install.sh if there is no PSfilter in ./LPsource

directory.

lpsource subdirectory

---------------------

The lpsource subdirectory contains the following files:

--PS.aix41ppc, PS.sco3, PS.sco5, PS.sun5

These four executable files are provided in case there is no C Compiler in your

AIX 4.1 Power PC, SCO 3.x, SCO 5.x or SUN 5.x UNIX system.  To use any of these

four programs, you need to copy the program file to a new file called "PSfilter"

in the LPsource directory.

--psfilter.c, errormsg.h

psfilter.c & errormsg.h are the source files used to generate the PSfilter

executable program.

PSfilter resides in the LPsource directory by default.  PSfilter can be used as

a command line utility to print the print jobs through PrintServer.  PSfiler can

also be invokded by UNIX's lp deamon in the background.  PSfilter provides many

options to meet the needs of printing a print job.  PSfilter takes the following

options:

-D destination  Send data file to destination.  Destination is the host name

of PrintServer.  This option must be specified.

-P              Data is sent to parallel port 2 of PrintServer.

-o3             Selected print port is parallel port 3.

(If -P or -o3 are not specified, parallel port 1 will be

used.)

-p string       Send string before a print job to initialize the printer.

If blank spaces are in the string, quotes must be used to

identify the string.

-s string       Send string after a print job to reset the printer.

If blank spaces are in the string, quotes must be used to

identify the string.

-h head_file    Send head_file before a print job to initialize the printer.

Head file must be specified with full path.

-t tail_file    Send tail_file after a print job to reset the printer.

Tail file must be specified with full path.

-d delay_min    The number of minutes allowed for the occurrence of the printer

faults before the active print job is aborted. (Default value of

delay minutes is 0.)

-a AnotherPort  Automatically send the print job to AnotherPort if the selected

printer is busy.

AnotherPort can be:

para1           Parallel port 1.

para2           Parallel port 2.

all             The other two ports.

-T              Convert LF in the print job to CR/LF.

Note: If both pre_string and head_file are specified, the pre_string is sent

before the head_file.  Similarly, the tail file is sent before the

suf_string.

With the following commands, you can send a file to parallel port 1,

parallel port 2 or parallel port 3 respectively ( provided the PrintServer is

installed and its IP address is set).

PSfilter -D PS_name < file_name

PSfilter -D PS_name -P < file_name

PSfilter -D PS_name -o3 < file_name

*************** How to do a Quick PrintServer Installation ***************

1. Connect print cables and network cable to your PrintServer.

2. Connect power adapter to your PrintServer to power up the PrintServer.

3. Set the IP address of the PrintServer using rarp or bootp.

Reset the PrintServer so it will acquire the IP Address you assigned.

4. Go to the directory where the utilities will be installed.  Insert the

CD into your CD-ROM device and type the command:

tar xvf CD-ROM mounted directory/Not_supp/driver/lptitar/lpti.tar

5. Change to the ./LPTI directory created by the previous command and

execute install.sh.

e.g.

cd LPTI

./install.sh  <parameters>

6. On system V, print files by the following command:

lp -d printer_name  file_names

On BSD system , print files by the following command:

lpr -P printer_name  file_names