BR6004W-G2M
Setup Wizard for VirginMedia/Cable customers

If you are running Operating Systems other than Microsoft® Windows XP® please refer to the ‘User Manual’ accompanied with this DYNAMODE Broadband Router!

NOTE: PLEASE MAKE SURE YOUR NEW DYNAMODE ROUTER, CABLE MODEM AND COMPUTER ARE ALL POWERED OFF BEFORE PROCEEDING WITH THIS INSTALLATION


Fig 1 (rear view)

STEP 1 – CONNECTTING YOUR NEW ROUTER TO YOUR CABLE MODEM & COMPUTER

Connect the supplied CAT5 network cable to the WAN port at the rear of the router (far left RJ45 socket if looking from the rear, see Fig 1) and connect the other end of the supplied CAT5 network cable to your existing DSL/Cable modem.

Connect a second CAT5 network cable to your Router’s LAN Ports (1~4) and the other end to your
existing Computer’s Network Interface (Network RJ45 port)

STEP 2 – POWER UP THE ATTACHED DEVICES IN THIS ORDER

1. DYNAMODE BROADBAND BR6004W-G2M WIRELESS ROUTER

│

2. YOU’RE CABLE/DSL MODEM

│

3. EXISTING COMPUTERS, LAPTOPS CONNECTED TO THIS 
DYNAMODE BROADBAND ROUTER

(Note: When the Dynamode broadband router is powered up the POWER light, WAN light,
M1 light, Switch Port (1~4) light and WLAN light will glow or flash.

If they do not please check the Power Supply has been correctly installed on the Router and connected devices. 
Please refer to the User Manual for status LED descriptions and diagnostics

STEP 3 – LOGIN TO YOUR DYNAMODE BROADBAND ROUTER

Open up Internet Explorer or your preferred Web Browser and type the following in the address field

192.168.1.1 { Press Enter }

wait for a dialog box to appear – this will ask you for a Username and Password…

Enter the default USERNAME admin PASSWORD admin

Note: If you are unable to get the initial Router Setup login Prompt check the following
 Internet Explorer (IE) settings:-

Click ‘TOOLS’-‘INTERNET OPTIONS’-‘click on the CONNECTTIONS Tab-
Under Dial up and virtual network settings

Click ‘NEVER DIAL A CONNECTION’ Radio Button -

‘Click’ OK button to save changes and then close Internet Explorer and start step 5 again.

(Note: if you still can not login to the DYNAMODE Router please move to STEP 4 and if you can login to the DYNAMODE Router please skip STEP 4 and move onto STEP 5 to finish the router setup)

STEP 4 – CONFIGURING NETWORK & INTERNET CONNECTIONS

Click START under Windows® XP → CONTROL PANEL → NETWORK CONNECTIONS

’Right Click’ LOCAL AREA CONNECTION → select PROPERTIES

In the  ‘GENERAL’ Tab under ‘This connection uses the following items’ scroll down to highlight ‘INTERNET PROTOCOL (TCP/IP) → ‘Click’ on PROPERTIES button

‘Click’ OBTAIN AN IP ADDRESS AUTOMATICALLY 
(Note: This is very important!)

‘Click’ OBTAIN DNS SERVER ADDRESS AUTOMATICALLY 
(Note: This is very important!)

‘Click’ OK button to save changes

‘Click’ OK again

Please start STEP 3 again

STEP 5 – RUNNING THE BUILT-IN SETUP WIZARD FOR THE FIRST TIME

Once correctly logged in with the correct Username and Password you will presented with the main setup page

Click ‘WIZARD’ button at the Top Left of the Main Setup Page

(1)  HOST SETTINGS		                                         (2) WAN SETTINGS – SPECIFY WAN CONNECTION
          Enter your Time Zone, Daylight savings..                                         Select ‘Cable Modem’ if connected thru a Cable Modem *

(3) DNS SETTINGS			                            (4) Click ‘FINISH’ – displays OK! (at top of screen)
          Input your DNS Settings if provided by ISP! *                                                then Click ‘Continue’ to show ‘System Status’
            otherwise leave blank

* Please consult with your ISP if unsure about specific Setup requirements
Please refer also to Installation Guide regarding detailed Setup and Wireless configuration