Firmware Release Note

1. Product name: 	WRT54GL

2. Product Version: 	V1

3. File detail:

Firmware Version:	v4.30.11

Release Date:		August 17, 2007

4. Summary of changes:

4.30.11

012

1. Add: DHCP IP Subnet Mask Automation feature (Sync from WRTSL54GS and WRT150N).

011

1. Fix: Some GUI issues (Issue id 8277, 8279, 8280).

2. Fix: DUT doesn't update DDNS in 10 min if the client gets no reply from DynDNS server issue (Issue id 8179).

3. Fix: DUT can't display response codes issue (Issue id 8181).

4. Fix: DUT send packets to echo server every 400 seconds, it should be 600 seconds issue (Issue id 8178).

5. Fix: DUT doesn't pop up warming message and allows be saved if key in wrong format of TZO key and domain name issue (Issue id 8177).

6. Fix: Configure DUT to be L2TP with Connect on demand in 5 minutes, but DUT will not disconnect L2TP after 5 minutes issue.

7. Fix: DUT doesn't respond a L2TP ZLB Control Message to L2TP server issue.

010

1. Fix: No UPnP icon is shown in ‘My Network Places’ issue (Issue id 7564).

2. Fix: When DUT LAN IP is in DHCP pool, DHCP server can not assign the correct IP addresses set on web page (Issue id 7597).

3. Fix: DUT doesn't send 3 gratuitous ARP packets in DHCP and Static IP mode issue (Issue id 7571).

4. Fix: When DUT without a WAN IP, open ping test window and enter target IP address as 192.168.1.1. Ping test doesn't get any reply. It shows "Network is unreachable" (Issue id 7630).

5. Fix: DUT sends only one IGMP V2 general query message to LAN subnet after rebooting (Issue id 7589).

6. Fix: DUT sends only one IGMP group-specific query to LAN subnet when VLC client sends IGMP Leave message (Issue id 7590).

7. Fix: DUT can not send IGMP V1 queries to LAN subnet anytime when sending IGMP V1 report message to join group or leave group from LAN PC (Issue id 7679).

8. Fix: Run Chariot transmitted traffic from PC1 (low), PC2 (low) to WAN side with as longer duration 10 min. Change the PC1_MAC priority to Highest at 2 min, but do not stop chariot. Change the PC2_MAC priority to High at 4 min. At 6 min, disable QOS totally. When disable QOS totally, DUT often crash (Issue id 7684).

9. Fix: Run Chariot transmitted traffic from PC1 (23/TCP), PC2 (80/TCP) and PC3 (65535/TCP) to Wan side with as longer duration 10 min. Change the PC1 (23/TCP) priority to Highest at 2 min, but do not stop chariot. Change the PC2 (80/TCP) priority to High at 4 min. At 6 min, disable QOS totally. When disable QOS totally, DUT often crash (Issue id 7693).

009

1. Fix: PPTP can not trigger issue (For Setup Wizard test).

008

1. Fix: PPTP can not connect to Linux PPTP server issue (For Setup Wizard test).

007

1. Fix: The same MAC or the same ports can be accepted on web page ‘QoS.asp’ issue (Issue id 7682, 7692).

2. Fix: PPTP pass-through can not work with WRT54Gv8 issue.

006

1. Fix: Some spelling errors in help pages (Issue id 7562).

2. Fix: Some GUI issues (Issue id 7551, 7552, 7563, 7576, 7577, 7578).

3. Fix: IPSec pass-through can not support multi-IPSec issue (Issue id 7595).

4. Fix: DDNS would not send packet with interval 10 minutes issue (Issue id 7588).

5. Fix: The default metric is not correct in routing table (Issue id 7580).

6. Fix: The metric for the route is not set to 16 (infinity) if no RIP received in 180 seconds (Issue id 7581).

7. Fix: The RIP function would receive the RIP packets with a wrong version (Issue id 7582).

004

1. Fix: CDRouter v3.6 ICMP items fail issue.

003

1. Fix: Downgrade F/W code lead to web page display in an abnormal form.

000

1. Fix: Policy Type is unavailable in Internet Access, remove the help info on the page.

4.30.10

002

1. Fix: Disable Wireless Function and click "Reset Security" button makes wireless function not working issue.

2. Update: Daylight Saving Time policy for USA is updated.

001

1. Fix: Can not rewrite MAC and SN issue.

000

1. Fix: UPnP security issue.

2. Update TZO with latest protocol.

4.30.9

002

1. Fix: L2TP connecting failed issue.

001

1. Fix: Linux kernel vulnerable issue of race condition in netfilter.

2. Fix: CDRouter V3.3 UPnP issues.

- Item 209   cdrouter_upnp_200, Verify UPnP clients can subscribe/unsubcribe to events for WANIPConnection or WANPPPConnection;

- Item 210 cdrouter_upnp_201, Verify UPnP clients can subscribe to events with infinite subscription time;

- Item 211 cdrouter_upnp_202, Verify UPnP clients can renew NOTIFY events for WANIPConnection or WANPPPConnection;

- Item 212 cdrouter_upnp_203, Verify router sends UPnP NOTIFY events for ConnectionStatus;

- Item 213 cdrouter_upnp_204, Verify router sends UPnP NOTIFY events with updated ExternalIPAddress;

- Item 215 cdrouter_upnp_220, Verify the maximum number of UPnP event subscriptions that can be created

000

1. First maintenance release by GZ team.

2. Sync Access Restriction Policy with WRT54G V5.