DVS5G5 SD card update FW process

- SD Card update

- Format the SD card

- Copy dvs5g5.bin to SD card and insert it to the DV

- Press down Shutter key and hold

- Power on the DV.

(If battery is full, it can auto run FW update)

5.  During updating process, the led is on

6.  After updated complete, the led is off. Then DV will power off automatically.

The new DVS5G5 FW is running when next time power on.

- SD 卡更新

- 格式化SD卡

- 拷貝dvs5g5.bin檔案至SD卡，并將其插入相機。

- 按下Shutter鍵,并保持不放。

- 開機（如電池滿電，相機會自動運行FW更新）。

- 更新時，LED指示燈會亮。

- 更新完成，LED燈熄滅，并自動關機。

下次開機，會自動運行新的FW.